| Phone Functions Wireless Characteristic Signal type: 802.11b/g Data Rate: 1, 2, 5.5, 6, 11, 12, 22, 24, 54 Mbps Operating Frequencies: - USA/Canada: 2.412MHz-2.462 MHz (11 channels) - Europe: 2.412-2.472 GHz (13 channels) - Japan: 2.412-2.472 GHz and 2.484 GHz (14 channels).. Ring tone/Vibrator switch Key lock Pre-dial before sending Handset / Speakerphone Volume adjustment Speed dial (10 records) Phone book (200 records) Call history (Incoming calls / Outgoing calls / Missed calls) Advanced Applications JAVA virtual machine for JAVA applications JAVA OTA supported for applications download Organizer Tools Message Dial Methods Direct IP call without SIP registration Dial registered number via SIP server Dial URI from phone book / speed dial Power Supply Rechargeable battery: 3.7V, 1050mAh Li-Polymer Talking time: 4 hours Standby time: 70 hours RF output power: 32mW Physical Dimensions Size: 105(L) x 45(W) x 19(H) mm Weight: 100g Color: Silver | VoIP Protocol IETF SIP (RFC3261) LCD Display LCD Type: TFT-LCD Display size (Diagonal): 1.8 inch Display Color: 262.144 colors Format: 132RGB x 176-dot graphics display on TFT LCD Display type: Transflective Backlight: LED Key Pad 23 keys Connector Interface 18 pins connector for power charging and data transmitting Call Features Call Hold (melody or tone) Call Mute Call Retrieve Call Transfer (Unattended / Blind & Attended) Call Waiting Call Forward (Busy / No Answer /Unconditional) Caller ID Display Caller ID Blocking Anonymous Call Blocking Out-of-band DTMF (RFC 2833) Message Waiting Indicator 3-way Conference Redial Full duplex Speaker Phone Codec G.711µ-law, G.711a-law, G.723.1 (5.3k), G.723.1 (6.3k), G.729a/b SIP Server Support Registrar Server (setting from web) Outbound Proxy (setting from web) Hardware AES Security HTTP 1.1 basic/digest authentication for Web setup MD5 for SIP authentication (RFC 2069/ RFC 2617) IEEE 802.1x, 802.11i Wired Equivalent Privacy (WEP) 64/128 bit encryption SIPS, sRTP continues next page  Voice Quality VAD (Voice Activity Detection) CNG (Comfort Noise Generation) AEC (Acoustic Echo Cancellation) G.168 Jitter buffer QoS ToS field Tone DTMF Ring Tone, 32 polyphonic tones +MP3 Ring Back Tone (local and remote) Dial Tone Busy Tone Key Tone (Beep/DTMF) IP Assignment Static IP DHCP PPPoE NAT Traversal UPnP STUN TCP/IP IP/TCP/UDP/DHCP/RTP/RTCP/IC MP/HTTP/SNTP/TFTP/DNS Configuration Key & LCD configuration Web browser configuration Auto/Manual provisioning system Firmware Upgrade TFTP / FTP / HTTP Auto/Manual provisioning system Power Management 802.11b/g compliant power management and enhanced power management Charger Input AC 110-220V Output DC 5V 1.4A Certification Compliance FCC Part 15 Class B CE Class B VCCI Class B, TELEC EN60950. |
| --- | --- |